TEST REPORT FOR INVERTER

MMA/TIG POWER SOURCE

MODEL	:

| MODEL | TICK () |
| --- | --- |
| CHAMP 400 X |  |

SLIP NO.	:							SR.NO.:

1.0  	SR.NO. OF PWM CONTROLLER CUM IGBT DRIVER PCB:

2.0	MECHANICAL & VISUAL CHECK	: OK / NOT OK

3.0	VISUAL INSPECTION:

| ENSURE FOLLOWING MECHANICAL ASPECTS | RESULT |
| --- | --- |
| 1) TIGHTNESS OF ALL HARDWARES OF MAIN TX. & IGBT ASSEMBLY | OK / NOT OK |
| 2) CRIMPING OF ALL LUGS | OK / NOT OK |
| 3) TIGHTNESS OF ALL HARDWARES | OK / NOT OK |

- CONTINUITY TEST :

A. ISOLATION BETWEEN IGBT HS & DIODE HS		      : OK/NOT OK

B. ISOLATION BETWEEN FABRICATED BODY & DIODE HS   : OK/NOT OK

C. ISOLATION BETWEEN FABRICATED BODY & IGBT HS      : OK/NOT OK

5.0 	DIELECTRIC (H.V.) TEST:

| ENSURE FOLLOWING ASPECTS | RESULT |
| --- | --- |
| - INPUT SUPPLY SHORT CIRCUIT(R,Y,B) & EARTH | OK / NOT OK |
| - OUTPUT TERMINAL SHORT CIRCUIT & EARTH | OK / NOT OK |
| - INPUT SUPPLY SHORT CIRCUIT(R, Y, B) & OUTPUT TERMINAL SHORT CIRCUIT | OK / NOT OK |

| Insulation Resistance Between | AC Dielectric Test Voltage applied for 5 sec | Current mA | Remarks |
| --- | --- | --- | --- |
| Input and Earth | 1.875 KV |  | OK/ NOT OK |
| Output And Earth | 1.875 KV |  | OK/ NOT OK |
| Input And Output | 3.750 KV |  | OK/ NOT OK |

6.0	WIRING CHECK AS PER DRG. NO. CK79/1174	                    : OK / NOT OK

7.0	FAN FLOW CHECK (BACK TO FRONT)			       : OK / NOT OK

8.0  	NO LOAD TEST: INPUT SUPPLY 415 VOLTS, 3 PHASE, 50 Hz

| NO LOAD CURRENT AMPS. | IR | IY | IB | ACCEPTANCE CRITERIA | RESULT |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | <= 1A | OK / NOT OK |
| OPEN CIRCUIT VOLTAGE (VOLTS) |  | OBSERVED VALUE |  | ACCEPTANCE CRITERIA | RESULT |
|  |  |  |  | 70V (+/-5V) | OK / NOT OK |

9.0 MAINS LED (GREEN) GLOWING		                         	        : OK / NOT OK

10.0 LOAD TEST: INPUT SUPPLY 415 VOLTS, 3 PHASE, 50 Hz.

| Sr. No. | PARAMETER | ACCEPTANCE VALUE | ACTUAL RESULT | REMARKS |
| --- | --- | --- | --- | --- |
| 1. | CONTROL TX. VOLTAGES | 18-0-18 +/-0.2VAC 0-9.5 +/-0.2VAC 0-9.5 +/-0.2VAC |  | OK / NOT OK |
| 2. | MIN. OUTPUT CURRENT | 10A @ 10V |  | ADJUST BY P3 POT |
| 3. | MAX. OUTPUT CURRENT | 400A @ 36V |  | OK / NOT OK |
| 4. | SHORT CKT. OUTPUT CURRENT WITH ARC FORCE SET AT MIN.(IN MMA MODE) | EQUAL TO 30A MORE THAN SET CURRENT FOR LESS THAN 3 SECOND |  | CHECK AT 100A SET CURRENT |
| 5. | SHORT CKT. OUTPUT CURRENT WITH ARC FORCE SET AT MAX.(IN MMA MODE) | 70-100A MORE THAN SET CURRENT FOR LESS THAN 3 SECOND |  | CHECK AT 100A SET CURRENT |
| 6. | SHORT CKT. OUTPUT CURRENT WITH ARC FORCE POT. AT MAX.(IN TIG MODE) | EQUAL TO 30A MORE THAN SET CURRENT |  | CHECK AT 100A SET CURRENT |
| 7. | SUPPLY VOLTAGE IMMUNITY @ 370VAC & 470VAC (IN MMA MODE) | EQUAL TO SET CURRENT +/- 5A |  | CHECK AT 200A SET CURRENT |

IMPORTANT: SEAL PRESETS P2, P3 POT BY SILASTIC AT THE EDGE ONLY.

DO NOT COVER THE WHOLE PRESET.

11.0      VRD UNIT TEST (In MMA Mode)			 	: OK / NOT OK

(Keep machine in no load condition.

Keep VRD switch at ON position.

check that OCV will become 6-8V after 3 min

Then do the welding and observe

that OCV will available after arc striking)

12.0	VRD UNIT TEST (In TIG Mode)				: OK / NOT OK

(Keep machine in no load condition.

Keep VRD switch at ON position.

OCV will be available.

VRD mode is not applicable.)

| MODEL | LOAD AMPS. / VOLTS | IR AMPS | IY AMPS | IB AMPS | I (Avg.) ACCEPT. CRITERIA | RESULT |
| --- | --- | --- | --- | --- | --- | --- |
| CHAMP 400 X | 400 / 36 |  |  |  | <= 26 | OK / NOT OK |
|  | 310 / 32 |  |  |  | <= 19 | OK / NOT OK |

13.0	SINGLE PHASING PROTECTION TEST			: OK / NOT OK

(Remove I/P Supply Phases one by one & check Err 001)

14.0	LOW VOLTAGE PROTECTION TEST	  (350Vac±10V)	: OK / NOT OK

(Check Err 001)

15.0	OVERVOLTAGE PROTECTION TEST (485Vac±10V)		: OK / NOT OK

(Check Err 002)

16.0	THERMAL PROTECTION TEST 				: OK / NOT OK

(Remove CN2 from Line Filter PCB & check the Err 003)

17.0	CONNECTION ERROR TEST 					: OK / NOT OK

(Remove CN9 of Display PCB & Check the Err 011)

18.0     SHORT CIRCUIT PROTECTION TEST (In MMA Mode)	: OK / NOT OK

(Load the machine at 100A, short the output & check that                                           output current goes to min. current -in 3-5 sec.)

19.0      ANTISTICK TEST (In MMA Mode)

(Load the machine at 100A, drop the o/p voltage below 10V         : OK / NOT OK

& check that output current goes to min. current.)

20.0	ANTISTICK TEST (In TIG Mode)				: OK / NOT OK

(Load the machine at 100A, short the output & check that                                           output current remains equal to set current.)

21.0      HOT START TEST (In MMA Mode)				: OK / NOT OK

22.0	REMOTE CONTROL OPERATION TEST			: OK / NOT OK

(Connect remote & check current variation by remote pot)

23.0	MMA WELDING TEST					: OK / NOT OK

24.0	TIG WELDING TEST						: OK / NOT OK

25.0	HEAT RUN TEST                  					: OK / NOT OK

Load the machine for 100% @ 310A / 32.4 V (MMA Mode)

For 45 min

TESTED BY:							                   DATE: